- Suppose a network using CSMA/CD has the bandwidth of 10 Mbps. If the maximum propagation is 20microsec. What will be the minimum frame size?

L=size of frame

BW-> Bandwidth=10Mbps=10*106 b/s

Tp -> Propagation time=20micro sec=20*10-6 s

Tt ->  Transmission time

Formula for CSMA/CD

Tt >  2 * Tp

Tt=L/BW

For frame size to be minimum, its transmission time should be equal to twice of one way propagation delay.

i.e, Tx = 2Tp

suppose minimum frame size is L bits.

Tx = L / B

where B is the bandwidth  = 20 * 106 bits/sec

and given Tp = 20 micro seconds

as Tx = 2 Tp

L / B = 40 micro seconds

L = B * 40 micro seconds = 20 * 106 bits/sec * 40 micro seconds = 800 bits

Now as the answer has to be given in Bytes, hence, 800 / 8 bytes = 100 bytes.

- A bit string, 0111101111101111110, needs to be transmitted

at the data link layer. What is the string transmitted after bit

stuffing?

- If the bandwidth of the line is 15 Mbps, RTT is 45ms and

packet size is 1 KB, then find the link utilization in Stop and

Wait Protocol

If the bandwidth of the line is 1.5 Mbps, RTT is 45 msec and packet size is 1 KB, then find the link utilization in stop and wait.

- Solution-

Given-

- Bandwidth = 1.5 Mbps

- RTT = 45 msec

- Packet size = 1 KB

- Calculating Transmission Delay-

Transmission delay (Tt)

= Packet size / Bandwidth

= 1 KB / 1.5 Mbps

= (210 x 8 bits) / (1.5 x 106 bits per sec)

= 5.461 msec

- In a slotted Aloha network with G = 1/2, how is the

throughput affected in each of the following cases? a. G is

increased to 1. b. G is decreased to 1/4.?

Show how the CRC codeword was generated at the sender site

using the dataword 101001111 and the divisor 10111.

Top of Form

Bottom of Form

Top of Form

Best answer

data word = 101001111
divisor =10111 ...here divisor is 5 bits so pad 4 0's in data word

10111 ) 101001111 0000
             10111
           -----------------------
            000111111 0000
                   10111
           ----------------------
                010001 0000
                  10111
            -----------------------
                  00110 0000
                      101 11
            -----------------------
                      011 1100
                         10 111
             ----------------------
                        01 0010
                          1 0111
                       --------------
                          0 0101

so code word will be 1010011110101

Bottom of Form

In a CSMA / CD network running at 1 Gbps over 1 km cable

With no repeaters, the signal speed in the cable is 200000 km/sec.

What is minimum frame size?

Top of Form

Bottom of Form

Top of Form

CSMA/CD can detect collisions always if below condition is satisfied:

T.T≥2∗P.T where T.T is Transmission time and P.T is Propagation time.

If above condition is satisfied CSMA/CD can work always and cannot stuck.

So, We know that T.T=Data size Bandwidth and P.T=Distance Velocity

Therefore,  DatasizeBandwidth⩾2∗DistanceVelocity

Datasize109⩾2∗10002∗108

by solving,  Data size ⩾ 10000 bits.

Hence, Option (A) is correct.

Bottom of Form

Assume that, in a Stop-and-Wait ARQ system, the bandwidth

of the line is 1 Mbps, and 1 bit takes 20 ms to make a round

trip. What is the bandwidth-delay product? If the system data

frames are 1000 bits in length , what is the utilization

percentage of the link?

The bandwidth-delay product is 1 × 10 6 × 20 × 10 -3 = 20,000 bits

The system can send 20,000 bits during the time it takes for the data to go from the sender to the receiver and then back again. However, the system sends only 1000 bits. We can say that the link utilization is only 1000/20,000, or 5%. For this reason, for a link with high bandwidth or long delay, use of Stop-and-Wait ARQ wastes the capacity of the link

Sixteen-bit messages are transmitted using hamming code. How many check bits are needed to ensure that the receiver can detect and correct

single-bit errors? Show the bit pattern transmitted for the message 1101001100110101. Assume that even parity is used in the Hamming code.

# Calculating the Hamming Code

The key to the Hamming Code is the use of extra parity bits to allow the identification of a single error. Create the code word as follows:

- Mark all bit positions that are powers of two as parity bits. (positions 1, 2, 4, 8, 16, 32, 64, etc.)

- All other bit positions are for the data to be encoded. (positions 3, 5, 6, 7, 9, 10, 11, 12, 13, 14, 15, 17, etc.)

- Each parity bit calculates the parity for some of the bits in the code word. The position of the parity bit determines the sequence of bits that it alternately checks and skips.
Position 1: check 1 bit, skip 1 bit, check 1 bit, skip 1 bit, etc. (1,3,5,7,9,11,13,15,...)
Position 2: check 2 bits, skip 2 bits, check 2 bits, skip 2 bits, etc. (2,3,6,7,10,11,14,15,...)
Position 4: check 4 bits, skip 4 bits, check 4 bits, skip 4 bits, etc. (4,5,6,7,12,13,14,15,20,21,22,23,...)
Position 8: check 8 bits, skip 8 bits, check 8 bits, skip 8 bits, etc. (8-15,24-31,40-47,...)
Position 16: check 16 bits, skip 16 bits, check 16 bits, skip 16 bits, etc. (16-31,48-63,80-95,...)
Position 32: check 32 bits, skip 32 bits, check 32 bits, skip 32 bits, etc. (32-63,96-127,160-191,...)
etc.

- Set a parity bit to 1 if the total number of ones in the positions it checks is odd. Set a parity bit to 0 if the total number of ones in the positions it checks is even.

Here is an example:

A byte of data: 10011010
Create the data word, leaving spaces for the parity bits: _ _ 1 _ 0 0 1 _ 1 0 1 0
Calculate the parity for each parity bit (a ? represents the bit position being set):

- Position 1 checks bits 1,3,5,7,9,11:
? _ 1 _ 0 0 1 _ 1 0 1 0. Even parity so set position 1 to a 0: 0 _ 1 _ 0 0 1 _ 1 0 1 0

- Position 2 checks bits 2,3,6,7,10,11:
0 ? 1 _ 0 0 1 _ 1 0 1 0. Odd parity so set position 2 to a 1: 0 1 1 _ 0 0 1 _ 1 0 1 0

- Position 4 checks bits 4,5,6,7,12:
0 1 1 ? 0 0 1 _ 1 0 1 0. Odd parity so set position 4 to a 1: 0 1 1 1 0 0 1 _ 1 0 1 0

- Position 8 checks bits 8,9,10,11,12:
0 1 1 1 0 0 1 ? 1 0 1 0. Even parity so set position 8 to a 0: 0 1 1 1 0 0 1 0 1 0 1 0

- Code word: 011100101010.

## Finding and fixing a bad bit

The above example created a code word of 011100101010. Suppose the word that was received was 011100101110 instead. Then the receiver could calculate which bit was wrong and correct it. The method is to verify each check bit. Write down all the incorrect parity bits. Doing so, you will discover that parity bits 2 and 8 are incorrect. It is not an accident that 2 + 8 = 10, and that bit position 10 is the location of the bad bit. In general, check each parity bit, and add the positions that are wrong, this will give you the location of the bad bit.

Consider a CSMA/CD network that transmits data at a rate of 100 Mbps (108  bits second) over a 1 km(kilometer) cable with no repeaters. If the minimum frame size required for this network is 1250 bytes, find what is the signal speed (km/sec) in the cable?

B = 108 bits/sec

d = 1 km

Road trip distance = 2 km

To detect a collision, sender must be transmitting when it receives the collision signal, so that it can transmit again.

∴ 1250 × 8/ 10 8 = 2 /s p e e d

∴ s p e e d = 2 × 10 8 /10 4 = 2 × 10 4

The message 1110001001 is to be transmitted using the CRC polynomial x^3+x^2 + 1 to protect it from errors. How the message should be transmitted and verify whether the message is accepted or rejected

Consider that the stop-and-wait protocol is used on a link with a bit rate of 128 Kbps and 40 msec as round-trip time. Assume that the transmission time for the acknowledgment and processing time at nodes is negligible. If the frame size is 512 B then find the link utilization in Percentage A slotted ALOHA network transmits 200-bit frames on a shared channel of 200 kbps.

What is the throughput if the system (all

stations together) produces.

a)10000 frames per second

b)5000 frames per second

c)200 frames per second.

In a fully-connected mesh network with 30 computers, calculate the

total numbers of cables are required for each device.

If efficiency is 50% and bandwidth is 4Mbps, calculate the

Throughput in stop and wait protocol.

Suppose a binary data 1001101 is to be transmitted. Calculate the

number of redundancy bits required using hamming

code.(R8R4R2R1)

1. Repeater a. Transport layer

2. Hub b. Data link layer

3. Bridge c. Physical layer

4. Switch d. Network layer

5. Router e. Application

layer

6. Transport

gateway

7. Application

Gateway

If the bandwidth of the line is 15 Mbps, RTT is 45ms and packet

size is 1 KB, then find the link utilization in Stop and Wait Protocol

A 1 Mbps satellite link connects two ground stations. The altitude

of the satellite is 36504 km and speed of the signal is 3 x

108 m/sec. What should be the packet size for a channel utilization

of 25% for a satellite link using go back 127 sliding window

protocol?

In a CSMA / CD network running at 1 Gbps over 10 km cable with

no repeaters, the signal speed in the cable is 200000 km/sec. What

is minimum frame size?

In CRC checksum method, assume that given frame for transmission

is 100110011011 and the generator polynomial is G(x) = x4 + x + 1.

After implementing CRC encoder, calculate the encoded word sent

from sender side and check for errors at the receiver side.

In Go Back 4 protocol and selective repeat protocol of Ws=3, if

every 6 th packet that is being transmitted is lost and if we have to

send 10 packets, then how many transmissions are required?

Consider a 10 Mbps Ethernet LAN that has stations attached to a 2.5

km long coaxial cable. Given that the transmission speed is 2.3 x

10 8  m/sec, the packet size is 128 bytes out of which 30 bytes are

overhead, find the effective transmission rate and maximum

rate/throughput at which the network can send data.

Explain about Ethernet Frame Format with a neat diagram.

Explain the three persistence methods of the CSMA protocol and

explain the working of Back off Algorithm.

What is polling?

What are the services and responsibilities of Data

Link Layer?

What are Controlled Access Protocols and

Explain any two Protocols ?

A Go-Back-N ARQ protocol uses a 4-bit

sequence number and a sender window size of

10. The sender transmits frames 0 through 9, but

Frame 4 is lost while Frames 5–9 reach the

receiver.

Determine:

The total number of sequence numbers.

Whether a window size of 10 is valid.

What happens to Frames 5–9 at the receiver.

Which frames the sender retransmits.

A shared Ethernet network of length 250 meters

operates at 10 Mbps. A computer wants to send

a 1000-byte frame. The signal propagation

speed in the cable is 2 × 10^8 m/s. (a). Calculate

the propagation delay. (b) Calculate the

transmission time for the frame.

a. sender wants to transmit the 7-bit data

1011001 using Hamming (7,4) code.Determine

the Hamming code that should be transmitted

(i.e., calculate the parity bits). Suppose the

received code is 1111001. Identify if there is

an error, and if so, correct it. Hints: Use even

parity for Hamming code.

3 3 1 1,2

b. A sender and receiver are using Selective

Repeat sliding window protocol with a window

size of 4. The sender needs to transmit 8

frames (F0 to F7). During transmission, frame

F2 and frame F5 are lost.

A)Show the order of frame transmission and

acknowledgment. B)Explain how the sender

retransmits only the lost frames.C)How many

frames are sent before waiting for

acknowledgment, assuming the window is

fully utilized?Given: Window size = 4 and

Total frames = 8

A LAN uses CSMA/CD (Carrier Sense

Multiple Access with Collision Detection) to

control access to a shared Ethernet channel.

Suppose 5 computers are connected to the

network.Explain, step by step, the CSMA/CD

algorithm for sending a frame. Suppose two

computers attempt to transmit at the same time,

illustrate how the collision is detected and

resolved.

Calculate the minimum frame size if the

network length is 500 meters and the

propagation speed is 2 × 10^8 m/s. The data

rate is 10 Mbps.